Digital Design COEN 6501                                           Midterm exam. Oct 17th , 2016

Answer all questions.                                                    Time allowed 1 hr 30 min.

All questions carry equal mark                                     A. J. Al-Khalili.

Question 1:

Using array multipliers give an optimum circuit for calculation of Y:

Y= [4 N ( N ) ]  - 1

where N is a 3-bit unsigned binary number. N = a2 a1 a0

Show your implementation with N = 2.

Give a detailed estimate of delay and area (Full Adder, Half Adder AND gate or other entity used).

Show the input to each adder clearly.

Question 2:

You are to design a 9 bit adder. You have two choices to design the adder:  either a Carry Ripple Adder or a Carry Select Adder. Your design Criterion is (A.T, The area time product) where A, is the Area and T is the time delay. You are given the following parameters:

Full Adder Delay= TFA         Full Adder Area = AFA

MUX Delay = ½  TFA          MUX   Area = 1/3 AFA

Give your optimum Design.

Question 3:

Multiply The following floating point numbers A * B

A =  21.7510         B = -7.2510

Give your final result in a packed format. Use IEEE 754 format.