WAVES Specification Data:

Additional information on WAVES is available in hardcopy. The

WAVES specification and supporting documents are under constant

development to support the standardization of WAVES by the

IEEE. This documentation set includes detailed explanations of

the WAVES data structures, VHDL WAVES Object types, and

examples. This data is only available in hardcopy form due to

the intermix of text and  graphics.

For a copy of the current specification please send your

requests to:

WAVES Analysis Standardization Group

c/o Robert Hillman

Rome Air Development Center

MS:RBRP

Griffiss Air Base

Rome, NY 13441-5700

(note in your request, if you desire to be on the WAVES mailing

list for future events and data.)

--**************************************************************************************************

This file contains the following information:

1.1) Waves_Standard_.vhd  - package spec

1.2) Waves_Standard.vhd   - package body

2.1) Waves_Interface_.vhd - package spec

2.2) Waves_Interface.vhd  - package body

3.1) Waves_Port_.vhd      - package spec

3.2) Waves_Port.vhd       - package body

4.1) Waves_Objects_.vhd   - package spec

4.2) Waves_Objects.vhd    - package body

5.1) Example Waves_Generator.wav  package spec and body

Example are vectors for a TTL full adder from

Intermetrics  3 day VHDL hands on course.

The files begin

--**************************************************************************************************

--

-- Copyright 1989 by ---

--

-- This code is distributed for the purposes of evaluating the

-- Waveform And Vector Exchange Specification (WAVES) proposal

-- presented to the IEEE by the WAVES Analysis Group.  This code

-- may not be used for commercial purposes and may not be

-- redistributed without permission of the Chairman of the WAVES

-- Analysis Group, Mr Robert Hillman.

--

-- Address comments or questions to

--    Robert Hillman                   Lee Shombert

--    RADC/RBRP                        Harris Corporation

--    Griffis AFB, NY                  PO Box 94000  MS 16/4010

--    (315) 330-2241                   Melbourne, FL 32902

--                                     (407) 727-6040

--

--

--

--

package WAVES_STANDARD is

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- These definitions provide the basic components of a simulation value.

-- The order of the declarations is significant, since variables of

-- these types will initialized to UNSPECIFIED.  UNSPECIFIED means that

-- information for that component was not supplied.  UNKNOWN, on the

-- other hand, indicates that the component could take on any possible

-- value greater than UNKNOWN (UNKNOWN does not include UNSPECIFIED nor

-- itself).  The exception is DIRECTION, for which UNKNOWN means either

-- INPUT or OUTPUT, but not COMPOUND.

--

-- DIRECTION indicates whether the waveform is forcing a value onto a

-- node (INPUT) or predicting a value on a node (OUTPUT) or both

-- (COMPOUND).

--

-- RELEVANCE indicates whether the waveform value is significant or

-- whether it may be an artifact of the waveform construction.

--

-- STATE indicates logic level.

--

-- STRENGTH supports bus resolution.

--

type DIRECTION is (

UNSPECIFIED, UNKNOWN, INPUT, OUTPUT, COMPOUND );

type RELEVANCE is (

UNSPECIFIED, UNKNOWN, MASK, CARE );

type STATE     is (

UNSPECIFIED, UNKNOWN, LOW, MIDBAND, HIGH );

type STRENGTH  is (

UNSPECIFIED, UNKNOWN, DISCONNECTED, CAPACITIVE,

RESISTIVE, DRIVE, SUPPLY );

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The STRENGTH RANGE record stores a lower and upper bound that may be

-- associated with a STRENGTH component.  The bounds may be left

-- unspecified by setting the SPECIFIED field appropriately.  Note that

-- variables of this type initialize to not specified.

--

type STRENGTH_RANGE is record

SPECIFIED             : BOOLEAN;

LOWER_BOUND           : REAL;

UPPER_BOUND           : REAL;

end record;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- These two ranges are defined here for convenience in creating the

-- specification.  They may be replaced everywhere by their definitions

-- at any time.  They are not necessarilly part of WAVES.

--

subtype STA_RNG is STATE

range STATE'SUCC(UNKNOWN) to STATE'RIGHT;

subtype STR_RNG is STRENGTH

range STRENGTH'SUCC(UNKNOWN) to STRENGTH'RIGHT;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following definitions build up to the definition of a EVENT_VALUE.

--

-- A LOGIC_VALUE_FLAG stores a bit flag and a strength range.  The

-- INCLUDED field indicates whether an element of any of the three

-- arrays that comprise a EVENT_VALUE (see below) is a possible condition

-- of the EVENT_VALUE.  If INCLUDED is TRUE, then STRENGTH_VALUE is

-- significant.

--

-- STATE_STRENGTH array is a two dimensional array that indicates

-- whether a state, strength combination is selected for a EVENT_VALUE.

--

-- STATE_NO_STRENGTH array is a one dimensional array that indicates

-- whether a state with unspecified strength is selected for a EVENT_VALUE.

--

-- STRENGTH_NO_STATE array is a one dimensional array that indicates

-- whether a strength with unspecified state is selected for a EVENT_VALUE.

--

-- SIMPLE_LOGIC_VALUE collects the three arrays above along with a

-- relevance field.

--

type LOGIC_VALUE_FLAG is record

INCLUDED              : BOOLEAN;

STRENGTH_VALUE        : STRENGTH_RANGE;

end record;

type STATE_STRENGTH_ARRAY is array ( STA_RNG, STR_RNG )

of LOGIC_VALUE_FLAG;

type UNSPECIFIED_STRENGTH_ARRAY is array ( STA_RNG )

of LOGIC_VALUE_FLAG;

type UNSPECIFIED_STATE_ARRAY is array ( STR_RNG )

of LOGIC_VALUE_FLAG;

type SIMPLE_LOGIC_VALUE is record

STATE_NO_STRENGTH   : UNSPECIFIED_STRENGTH_ARRAY;

STRENGTH_NO_STATE   : UNSPECIFIED_STATE_ARRAY;

STATE_STRENGTH      : STATE_STRENGTH_ARRAY;

LOGIC_RELEVANCE     : RELEVANCE;

end record;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- A EVENT_VALUE lists a set of possible waveform values.

-- If EVENT_DIRECTION is UNSPECIFIED, then the possible node values are

-- taken from INPUT_LOGIC and OUTPUT_LOGIC with no direction implied.

-- If EVENT_DIRECTION is INPUT or OUTPUT then the possible node values

-- are taken from the appropriate blocks.  If EVENT_DIRECTION is UNKNOWN

-- then possible input values are taken from INPUT_LOGIC and possible

-- output values from OUTPUT_LOGIC.  If EVENT_DIRECTION is COMPOUND then

-- possible input values are taken from INPUT_LOGIC and possible output

-- values from OUTPUT_LOGIC, except that one from each side is applied

-- simultaneously.

--

type EVENT_VALUE is record

EVENT_DIRECTION     : DIRECTION;

INPUT_LOGIC         : SIMPLE_LOGIC_VALUE;

OUTPUT_LOGIC        : SIMPLE_LOGIC_VALUE;

end record;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This is the type of the required WAVES object VALUE DICTIONARY.

--

type EVENT_VALUE_LIST is array (POSITIVE range <>) of EVENT_VALUE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following three functions make it easy to specify a event values.

-- The first three are used to specify event values for any direction

-- other than COMPOUND.  The fourth is for COMPOUND event values.

--

-- The first function allows all fields to be defaulted to UNSPECIFIED.

--

-- The second function allows the specification of a single real

-- modifier for strength.

--

-- The third function allows the specification of a range modifier for

-- strength.

--

function EVALUE (

S_STATE               : STATE      := UNSPECIFIED;

S_STRENGTH            : STRENGTH   := UNSPECIFIED;

S_DIRECTION           : DIRECTION

range UNSPECIFIED to OUTPUT    := UNSPECIFIED;

S_RELEVANCE           : RELEVANCE  := UNSPECIFIED )

return EVENT_VALUE;

function EVALUE (

S_STATE               : STATE;

S_STRENGTH            : STRENGTH;

RANGE_SINGLE          : REAL;

S_DIRECTION           : DIRECTION

range UNSPECIFIED to OUTPUT    := UNSPECIFIED;

S_RELEVANCE           : RELEVANCE  := UNSPECIFIED )

return EVENT_VALUE;

function EVALUE (

S_STATE               : STATE;

S_STRENGTH            : STRENGTH;

RANGE_LOWER           : REAL;

RANGE_UPPER           : REAL;

S_DIRECTION           : DIRECTION

range UNSPECIFIED to OUTPUT    := UNSPECIFIED;

S_RELEVANCE           : RELEVANCE  := UNSPECIFIED )

return EVENT_VALUE;

function COMPOUND_EVALUE (

I_NODE                : EVENT_VALUE;

O_NODE                : EVENT_VALUE )

return EVENT_VALUE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The first function performs the set addition of two event values.

-- The next function takes two event values as corners of a hypercube

-- and returns the filled in hypercube.  The next function fills

-- unspecified fields in the first parameter with data from the second

-- parameter.

--

function ADD_EVALUE (

A                   : EVENT_VALUE;

B                   : EVENT_VALUE ) return EVENT_VALUE;

function SUBTRACT_EVALUE (

A                   : EVENT_VALUE;

B                   : EVENT_VALUE ) return EVENT_VALUE;

function EVALUE_RANGE (

A                   : EVENT_VALUE;

B                   : EVENT_VALUE ) return EVENT_VALUE;

function MERGE_EVALUE (

A                   : EVENT_VALUE;

B                   : EVENT_VALUE ) return EVENT_VALUE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- An EVENT_TIME consists of a nominal time and an earliest and latest

-- possible deviation.  The EARLIEST and LATEST fields are only

-- significant if TOLERANCES_SPECIFIED is TRUE.

--

type EVENT_TIME is record

NOMINAL               : TIME;

TOLERANCES_SPECIFIED  : BOOLEAN;

EARLIEST              : TIME;

LATEST                : TIME;

end record;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following three functions make it easy to declare an EVENT_TIME.

--

-- The first function returns an event time with no tolerances.  The

-- nominal time defaults to 0 ns.

--

-- The second function returns an event time with symmetric bounds.

--

-- The third function allows the bounds to be specified independently.

--

function ETIME (

NOMINAL            : TIME := 0ns )

return EVENT_TIME;

function ETIME (

NOMINAL             : TIME;

EARLIEST_AND_LATEST : TIME )

return EVENT_TIME;

function ETIME (

NOMINAL            : TIME;

EARLIEST           : TIME;

LATEST             : TIME )

return EVENT_TIME;

function MERGE_ETIME (

A                  : EVENT_TIME;

B                  : EVENT_TIME )

return EVENT_TIME;

end WAVES_STANDARD;

**************************************************************************************************

--

-- Copyright 1989 by ---

--

-- This code is distributed for the purposes of evaluating the

-- Waveform And Vector Exchange Specification (WAVES) proposal

-- presented to the IEEE by the WAVES Analysis Group.  This code

-- may not be used for commercial purposes and may not be

-- redistributed without permission of the Chairman of the WAVES

-- Analysis Group, Mr Robert Hillman.

--

-- Address comments or questions to

--    Robert Hillman                   Lee Shombert

--    RADC/RBRP                        Harris Corporation

--    Griffis AFB, NY                  PO Box 94000  MS 16/4010

--    (315) 330-2241                   Melbourne, FL 32902

--                                     (407) 727-6040

--

--

-- This is the standard package used by all WAVES datasets

--

package body WAVES_STANDARD is

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- An internal function that sets a state/strength combination in a

-- simple logic value.  The old value of the logic value is passed in

-- so that settings are cummulative.

--

function UPDATE_LOGIC (

NEW_STATE        : STATE;

NEW_STRENGTH     : STRENGTH;

FLAG             : LOGIC_VALUE_FLAG;

OLD_LOGIC        : SIMPLE_LOGIC_VALUE )

return SIMPLE_LOGIC_VALUE

is

variable X         : SIMPLE_LOGIC_VALUE := OLD_LOGIC;

begin

if NEW_STATE = UNKNOWN then

for I in STA_RNG loop

X := UPDATE_LOGIC(I, NEW_STRENGTH, FLAG, X);

end loop;

elsif NEW_STRENGTH = UNKNOWN then

for J in STR_RNG loop

X := UPDATE_LOGIC(NEW_STATE, J, FLAG, X);

end loop;

elsif NEW_STATE = UNSPECIFIED and NEW_STRENGTH = UNSPECIFIED then

null;

elsif NEW_STATE = UNSPECIFIED then

X.STRENGTH_NO_STATE ( NEW_STRENGTH ) := FLAG;

elsif NEW_STRENGTH = UNSPECIFIED then

X.STATE_NO_STRENGTH ( NEW_STATE ) := FLAG;

else

X.STATE_STRENGTH ( NEW_STATE, NEW_STRENGTH ) := FLAG;

end if;

return X;

end UPDATE_LOGIC;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This function returns a event value in which a single state/strength

-- intersection has been filled in.  As a special case, STATE and

-- STRENGTH can be UNKNOWN, which implies every value in STA_RNG and

-- STR_RNG, respectively.

--

function EVALUE (

S_DIRECTION         : DIRECTION

range UNSPECIFIED to OUTPUT;

S_STATE             : STATE;

S_STRENGTH          : STRENGTH;

S_RELEVANCE         : RELEVANCE;

RANGE_SPECIFIED     : BOOLEAN;

RANGE_LOWER         : REAL;

RANGE_UPPER         : REAL )

return EVENT_VALUE

is

variable FLAG         : LOGIC_VALUE_FLAG :=

(TRUE, (RANGE_SPECIFIED, RANGE_LOWER, RANGE_UPPER));

variable LOGIC        : SIMPLE_LOGIC_VALUE;

variable DATA         : EVENT_VALUE;

begin

LOGIC := UPDATE_LOGIC(S_STATE, S_STRENGTH, FLAG, LOGIC);

LOGIC.LOGIC_RELEVANCE := S_RELEVANCE;

DATA.EVENT_DIRECTION := S_DIRECTION;

case DATA.EVENT_DIRECTION is

when UNSPECIFIED | UNKNOWN =>

DATA.INPUT_LOGIC := LOGIC;

DATA.OUTPUT_LOGIC := LOGIC;

when INPUT =>

DATA.INPUT_LOGIC := LOGIC;

when OUTPUT =>

DATA.OUTPUT_LOGIC := LOGIC;

when COMPOUND =>

null;

end case;

return DATA;

end EVALUE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following functions are the public interface to EVALUE.

--

function EVALUE (

S_STATE               : STATE      := UNSPECIFIED;

S_STRENGTH            : STRENGTH   := UNSPECIFIED;

S_DIRECTION           : DIRECTION

range UNSPECIFIED to OUTPUT    := UNSPECIFIED;

S_RELEVANCE           : RELEVANCE  := UNSPECIFIED )

return EVENT_VALUE

is

begin

return EVALUE (

S_DIRECTION, S_STATE, S_STRENGTH, S_RELEVANCE, FALSE, 0.0, 0.0 );

end EVALUE;

function EVALUE (

S_STATE               : STATE;

S_STRENGTH            : STRENGTH;

RANGE_SINGLE          : REAL;

S_DIRECTION           : DIRECTION

range UNSPECIFIED to OUTPUT    := UNSPECIFIED;

S_RELEVANCE           : RELEVANCE  := UNSPECIFIED )

return EVENT_VALUE

is

begin

return EVALUE (

S_DIRECTION, S_STATE, S_STRENGTH, S_RELEVANCE,

TRUE, RANGE_SINGLE, RANGE_SINGLE );

end EVALUE;

function EVALUE (

S_STATE               : STATE;

S_STRENGTH            : STRENGTH;

RANGE_LOWER           : REAL;

RANGE_UPPER           : REAL;

S_DIRECTION           : DIRECTION

range UNSPECIFIED to OUTPUT    := UNSPECIFIED;

S_RELEVANCE           : RELEVANCE  := UNSPECIFIED )

return EVENT_VALUE

is

begin

return EVALUE (

S_DIRECTION, S_STATE, S_STRENGTH, S_RELEVANCE,

TRUE, RANGE_LOWER, RANGE_UPPER );

end EVALUE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This function creates a compound event value out of two event values.

-- The input side of the first parameter is selected for the input side of

-- the result and the output side of the second parameter is selected for

-- the output side of the result.  If the first parameter is OUTPUT, then

-- the input side of the result is UNSPECIFIED.  If the second parameter

-- is INPUT then the output side of the result is UNSPECIFIED.

--

function COMPOUND_EVALUE (

I_NODE              : EVENT_VALUE;

O_NODE              : EVENT_VALUE )

return EVENT_VALUE

is

variable DATA : EVENT_VALUE;

begin

DATA.EVENT_DIRECTION := COMPOUND;

if I_NODE.EVENT_DIRECTION /= OUTPUT then

DATA.INPUT_LOGIC := I_NODE.INPUT_LOGIC;

end if;

if O_NODE.EVENT_DIRECTION /= INPUT then

DATA.OUTPUT_LOGIC := O_NODE.OUTPUT_LOGIC;

end if;

return DATA;

end COMPOUND_EVALUE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- An internal function that merges two strength ranges.  The result is

-- the envelope of the inputs.  That is, if both inputs are SPECIFIED

-- then the result runs from the minimum of the two lower bounds to the

-- maximum of the two upper bounds.  If only one input is SPECIFIED then

-- the result has the same bounds as that of the specified input.  If

-- neither is SPECIFIED then the result is not specified.

--

function MERGE_STRENGTH_RANGE (

A                   : STRENGTH_RANGE;

B                   : STRENGTH_RANGE )

return STRENGTH_RANGE

is

variable X : STRENGTH_RANGE;

begin

if A.SPECIFIED and B.SPECIFIED then

X.SPECIFIED := TRUE;

if A.LOWER_BOUND < B.LOWER_BOUND then

X.LOWER_BOUND := A.LOWER_BOUND;

else

X.LOWER_BOUND := B.LOWER_BOUND;

end if;

if A.UPPER_BOUND > B.UPPER_BOUND then

X.UPPER_BOUND := A.UPPER_BOUND;

else

X.UPPER_BOUND := B.UPPER_BOUND;

end if;

return X;

elsif A.SPECIFIED then

return A;

elsif B.SPECIFIED then

return B;

else

return (FALSE, 0.0, 0.0);

end if;

end MERGE_STRENGTH_RANGE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- An internal function that "or"'s two logic value flags.  If the

-- result of the "or" is an included logic value then the strength

-- ranges are merged.

--

function MERGE_LOGIC_FLAG (

A                   : LOGIC_VALUE_FLAG;

B                   : LOGIC_VALUE_FLAG )

return LOGIC_VALUE_FLAG

is

begin

if A.INCLUDED and B.INCLUDED then

return (

INCLUDED => TRUE,

STRENGTH_VALUE => MERGE_STRENGTH_RANGE (

A.STRENGTH_VALUE, B.STRENGTH_VALUE ) );

elsif A.INCLUDED then

return A;

elsif B.INCLUDED then

return B;

else

return (

INCLUDED => FALSE,

STRENGTH_VALUE => (FALSE, 0.0, 0.0) );

end if;

end MERGE_LOGIC_FLAG;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- An internal function that returns a normalized simple logic value.  A

-- normalized simple logic value does not have an unspecified state or

-- strength INCLUDED if there is a corresponding (state, strength)

-- combination INCLUDED.

--

function NORMALIZE_SIMPLE_LOGIC (

A                   : SIMPLE_LOGIC_VALUE )

return SIMPLE_LOGIC_VALUE

is

variable X            : SIMPLE_LOGIC_VALUE := A;

begin

for I in STA_RNG loop

for J in STR_RNG loop

if X.STATE_STRENGTH(I, J).INCLUDED then

X.STATE_NO_STRENGTH(I).INCLUDED := FALSE;

X.STRENGTH_NO_STATE(J).INCLUDED := FALSE;

end if;

end loop;

end loop;

return X;

end NORMALIZE_SIMPLE_LOGIC;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- An internal function that "or"'s two simple logic blocks together.

-- If both paramters have a state/strength combination set, then the

-- strength range selected is the envelope of the two strength ranges.

--

function MERGE_SIMPLE_LOGIC (

A                   : SIMPLE_LOGIC_VALUE;

B                   : SIMPLE_LOGIC_VALUE )

return SIMPLE_LOGIC_VALUE

is

variable X            : SIMPLE_LOGIC_VALUE;

begin

for I in STA_RNG loop

for J in STR_RNG loop

X.STATE_STRENGTH(I, J) := MERGE_LOGIC_FLAG (

A.STATE_STRENGTH(I,J), B.STATE_STRENGTH(I,J));

end loop;

end loop;

for I in STA_RNG loop

X.STATE_NO_STRENGTH(I) := MERGE_LOGIC_FLAG (

A.STATE_NO_STRENGTH(I), B.STATE_NO_STRENGTH(I));

end loop;

for J in STR_RNG loop

X.STRENGTH_NO_STATE(J) := MERGE_LOGIC_FLAG (

A.STRENGTH_NO_STATE(J), B.STRENGTH_NO_STATE(J));

end loop;

if A.LOGIC_RELEVANCE = B.LOGIC_RELEVANCE then

X.LOGIC_RELEVANCE := A.LOGIC_RELEVANCE;

elsif A.LOGIC_RELEVANCE = UNSPECIFIED then

X.LOGIC_RELEVANCE := B.LOGIC_RELEVANCE;

elsif B.LOGIC_RELEVANCE = UNSPECIFIED then

X.LOGIC_RELEVANCE := A.LOGIC_RELEVANCE;

else

X.LOGIC_RELEVANCE := UNKNOWN;

end if;

return NORMALIZE_SIMPLE_LOGIC(X);

end MERGE_SIMPLE_LOGIC;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- A function that adds ("or"'s) two event values.  Every state/strength

-- combination set in either inputs is set in the output.  If the same

-- combination is set in both inputs then the resultant strength range

-- is the envelope of the two input strength ranges.  The direction of

-- the result is determined as follows:

-- if either input is COMPOUND the result is COMPOUND

-- if either input is UNSPECIFIED the result is the direction of the

-- other input (including the case where both inputs are UNSPECIFIED).

-- if both inputs have the same direction, that is the direction of the

-- result.

-- otherwise the result has direction UNKNOWN.

--

function ADD_EVALUE (

A                   : EVENT_VALUE;

B                   : EVENT_VALUE )

return EVENT_VALUE

is

variable X            : EVENT_VALUE;

begin

X.INPUT_LOGIC := MERGE_SIMPLE_LOGIC (

A.INPUT_LOGIC, B.INPUT_LOGIC );

X.OUTPUT_LOGIC := MERGE_SIMPLE_LOGIC (

A.OUTPUT_LOGIC, B.OUTPUT_LOGIC );

if A.EVENT_DIRECTION = B.EVENT_DIRECTION then

X.EVENT_DIRECTION := A.EVENT_DIRECTION;

elsif A.EVENT_DIRECTION = COMPOUND or B.EVENT_DIRECTION = COMPOUND then

X.EVENT_DIRECTION := COMPOUND;

elsif A.EVENT_DIRECTION = UNSPECIFIED then

X.EVENT_DIRECTION := UNKNOWN;

X.INPUT_LOGIC := MERGE_SIMPLE_LOGIC (

X.INPUT_LOGIC, A.OUTPUT_LOGIC );

X.OUTPUT_LOGIC := MERGE_SIMPLE_LOGIC (

X.OUTPUT_LOGIC, A.INPUT_LOGIC );

elsif B.EVENT_DIRECTION = UNSPECIFIED then

X.EVENT_DIRECTION := UNKNOWN;

X.INPUT_LOGIC := MERGE_SIMPLE_LOGIC (

X.INPUT_LOGIC, B.OUTPUT_LOGIC );

X.OUTPUT_LOGIC := MERGE_SIMPLE_LOGIC (

X.OUTPUT_LOGIC, B.INPUT_LOGIC );

else

X.EVENT_DIRECTION := UNKNOWN;

end if;

return X;

end ADD_EVALUE;

-- A function that removes the event value tuples specified in the

-- second operand from those specified in the first operand.

-- Considering an event value as a set of tuples, the operation is

-- "A and not B".

--

function SUBTRACT_EVALUE (

A                   : EVENT_VALUE;

B                   : EVENT_VALUE )

return EVENT_VALUE

is

variable X            : EVENT_VALUE;

begin

return A;

end SUBTRACT_EVALUE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- An internal function that returns a simple logic value with (state,

-- strength) combinations filled in according to the single input.  The

-- input defines a hypercube.  The minimum strength and the minimum

-- state set in the input determine one corner of the hypercube.  The

-- maximum strength and the maximum state set in the input determine

-- the other corner.  The minimum lower bound used for any strength

-- range becomes the lower bound for all strength ranges between the two

-- corners and the maximum upper bound used for any strength range

-- becomes the upper bound for all strength ranges between the two

-- corners.  The three arrays STATE_STRENGTH, STATE_NO_STRENGTH, and

-- STRENGTH_NO_STATE are filled in separately, however the resulting

-- simple logic value is normalized before being returned.

--

function LOGIC_RANGE (

A                   : SIMPLE_LOGIC_VALUE )

return SIMPLE_LOGIC_VALUE

is

variable STA_MIN      : STATE    := UNSPECIFIED;

variable STA_MAX      : STATE    := UNSPECIFIED;

variable STR_MIN      : STRENGTH := UNSPECIFIED;

variable STR_MAX      : STRENGTH := UNSPECIFIED;

variable STR_FLAG     : STRENGTH_RANGE;

variable X            : SIMPLE_LOGIC_VALUE := A;

begin

for I in STA_RNG loop

for J in STR_RNG loop

if X.STATE_STRENGTH(I,J).INCLUDED then

if STA_MIN = UNSPECIFIED then

STA_MIN := I;

end if;

STA_MAX := I;

end if;

end loop;

end loop;

for J in STR_RNG loop

for I in STA_RNG loop

if X.STATE_STRENGTH(I,J).INCLUDED then

if STR_MIN = UNSPECIFIED then

STR_MIN := J;

end if;

STR_MAX := J;

STR_FLAG := MERGE_STRENGTH_RANGE(

STR_FLAG,

X.STATE_STRENGTH(I,J).STRENGTH_VALUE);

end if;

end loop;

end loop;

if STA_MIN /= UNSPECIFIED and STR_MIN /= UNSPECIFIED then

for I in STA_MIN to STA_MAX loop

for J in STR_MIN to STR_MAX loop

X.STATE_STRENGTH(I,J).INCLUDED := TRUE;

X.STATE_STRENGTH(I,J).STRENGTH_VALUE := STR_FLAG;

end loop;

end loop;

end if;

STA_MIN := UNSPECIFIED;

STR_FLAG := (FALSE, 0.0, 0.0);

for I in STA_RNG loop

if X.STATE_NO_STRENGTH(I).INCLUDED then

if STA_MIN = UNSPECIFIED then

STA_MIN := I;

end if;

STA_MAX := I;

STR_FLAG := MERGE_STRENGTH_RANGE(

STR_FLAG,

X.STATE_NO_STRENGTH(I).STRENGTH_VALUE);

end if;

end loop;

if STA_MIN /= UNSPECIFIED then

for I in STA_MIN to STA_MAX loop

X.STATE_NO_STRENGTH(I).INCLUDED := TRUE;

X.STATE_NO_STRENGTH(I).STRENGTH_VALUE := STR_FLAG;

end loop;

end if;

STR_MIN := UNSPECIFIED;

STR_FLAG := (FALSE, 0.0, 0.0);

for J in STR_RNG loop

if X.STRENGTH_NO_STATE(J).INCLUDED then

if STR_MIN = UNSPECIFIED then

STR_MIN := J;

end if;

STR_MAX := J;

STR_FLAG := MERGE_STRENGTH_RANGE(

STR_FLAG,

X.STRENGTH_NO_STATE(J).STRENGTH_VALUE);

end if;

end loop;

if STR_MIN /= UNSPECIFIED then

for J in STR_MIN to STR_MAX loop

X.STRENGTH_NO_STATE(J).INCLUDED := TRUE;

X.STRENGTH_NO_STATE(J).STRENGTH_VALUE := STR_FLAG;

end loop;

end if;

return NORMALIZE_SIMPLE_LOGIC(X);

end LOGIC_RANGE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- A function that returns a event value with state/strength combinations

-- filled in according to the two inputs.  The two input event values

-- represent corners of a hypercube.  The direction and relevance of the

-- result is the same as returned by the addition operation.

--

function EVALUE_RANGE (

A                   : EVENT_VALUE;

B                   : EVENT_VALUE )

return EVENT_VALUE

is

variable X            : EVENT_VALUE := ADD_EVALUE(A, B);

begin

X.INPUT_LOGIC := LOGIC_RANGE(X.INPUT_LOGIC);

X.OUTPUT_LOGIC := LOGIC_RANGE(X.OUTPUT_LOGIC);

return X;

end EVALUE_RANGE;

function MERGE_EVALUE (

A                   : EVENT_VALUE;

B                   : EVENT_VALUE )

return EVENT_VALUE

is

begin

return A;

end MERGE_EVALUE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following three functions make it easy to declare an EVENT_TIME.

--

-- The first function returns an event time with no tolerances.  The

-- nominal time defaults to 0 ns.

--

-- The second function returns an event time with symmetric bounds.

--

-- The third function allows the bounds to be specified independently.

--

function ETIME (

NOMINAL            : TIME := 0ns )

return EVENT_TIME

is

begin

return ( NOMINAL, FALSE, 0 ns, 0 ns );

end ETIME;

function ETIME (

NOMINAL             : TIME;

EARLIEST_AND_LATEST : TIME )

return EVENT_TIME

is

begin

return ( NOMINAL, TRUE, others => EARLIEST_AND_LATEST);

end ETIME;

function ETIME (

NOMINAL            : TIME;

EARLIEST           : TIME;

LATEST             : TIME )

return EVENT_TIME

is

begin

return ( NOMINAL, TRUE, EARLIEST, LATEST);

end ETIME;

function MERGE_ETIME (

A                  : EVENT_TIME;

B                  : EVENT_TIME )

return EVENT_TIME

is

variable X           : EVENT_TIME := A;

begin

if not X.TOLERANCES_SPECIFIED then

X.TOLERANCES_SPECIFIED := B.TOLERANCES_SPECIFIED;

X.EARLIEST := B.EARLIEST;

X.LATEST := B.LATEST;

end if;

return X;

end MERGE_ETIME;

end WAVES_STANDARD;

**************************************************************************************************

--

-- Copyright 1989 by ---

--

-- This code is distributed for the purposes of evaluating the

-- Waveform And Vector Exchange Specification (WAVES) proposal

-- presented to the IEEE by the WAVES Analysis Group.  This code

-- may not be used for commercial purposes and may not be

-- redistributed without permission of the Chairman of the WAVES

-- Analysis Group, Mr Robert Hillman.

--

-- Address comments or questions to

--    Robert Hillman                   Lee Shombert

--    RADC/RBRP                        Harris Corporation

--    Griffis AFB, NY                  PO Box 94000  MS 16/4010

--    (315) 330-2241                   Melbourne, FL 32902

--                                     (407) 727-6040

--

--

-- This is the standard package used by all WAVES datasets

--

library WAVES_LIBRARY;

use WAVES_LIBRARY.WAVES_STANDARD.all;

use WAVES_LIBRARY.WAVES_PORT.all;

package WAVES_INTERFACE is

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- An EVENT contains an integer selecting an EVENT VALUE from the

-- value dictionary and an EVENT TIME.

--

-- An EVENT_LIST is a list of EVENTs.

--

-- A TAGGED_EVENT is an EVENT with a flag indicating that the EVENT

-- really is not there.  The flag is the event reference equal to zero.

--

-- A FRAME is a list of TAGGED_EVENTs.  The frame "ends" at the end of

-- the array or at the first event that is unspecified.

--

-- A FRAME_SET is a two dimensional list of TAGGED EVENTS.  The first

-- index is meant to select frames and the second index is meant to

-- select (tagged) events within a frame.

--

-- A FRAME SET ARRAY is a three dimensional list of TAGGED EVENTS.  The

-- first index is meant to select frame sets.

--

type EVENT is record

VALUE                : POSITIVE;

TIME                 : EVENT_TIME;

end record;

type EVENT_LIST is array (POSITIVE range <>) of EVENT;

type TAGGED_EVENT is record

VALUE                : NATURAL;

TIME                 : EVENT_TIME;

end record;

type FRAME is array

(POSITIVE range <>)

of TAGGED_EVENT;

type FRAME_SET is array

(POSITIVE range <>, POSITIVE range <>)

of TAGGED_EVENT;

type FRAME_SET_ARRAY is array

(POSITIVE range <>, POSITIVE range <>, POSITIVE range <>)

of TAGGED_EVENT;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following functions are used by to create and manipulate frames,

-- frame sets, and frame set arrays.  The first set of functions return

-- the incoming object with it last index range adjusted to be (1 to L),

-- truncating or padding with unspecified tagged events, as appropriate.

-- The second set of operations are array concatenation, treating a

-- frame set as an array of frames and a frame set array as an array of

-- frame sets.

--

subtype FRAME_1 is FRAME;

subtype FRAME_2 is FRAME_SET;

subtype FRAME_3 is FRAME_SET_ARRAY;

function PROMOTE (

DATA               : FRAME_1 )

return FRAME_2;

function PROMOTE (

DATA               : FRAME_2 )

return FRAME_3;

function RESIZE (

DATA               : FRAME_1;

LENGTH_1           : POSITIVE )

return FRAME_1;

function RESIZE (

DATA               : FRAME_2;

LENGTH_2           : POSITIVE;

LENGTH_1           : POSITIVE )

return FRAME_2;

function RESIZE (

DATA               : FRAME_3;

LENGTH_3           : POSITIVE;

LENGTH_2           : POSITIVE;

LENGTH_1           : POSITIVE )

return FRAME_3;

function TRIM (

DATA               : FRAME_1 )

return FRAME_1;

function TRIM (

DATA               : FRAME_2 )

return FRAME_2;

function TRIM (

DATA               : FRAME_3 )

return FRAME_3;

function CONCAT(A, B : FRAME_1) return FRAME_1;

function CONCAT(A, B : FRAME_2) return FRAME_2;

function CONCAT(A, B : FRAME_3) return FRAME_3;

function "+"(A : FRAME_1;  B : FRAME_1) return FRAME_2;

function "+"(A : FRAME_2;  B : FRAME_1) return FRAME_2;

function "+"(A : FRAME_1;  B : FRAME_2) return FRAME_2;

function "+"(A : FRAME_2;  B : FRAME_2) return FRAME_2;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following functions convert a single event or a single event list

-- into a frame (a list of tagged events).  The empty case returns a

-- frame of length one with the tagged event marked unspecified.

--

function FRAME_EVENT

return FRAME;

function FRAME_EVENT (

EVENTS             : EVENT )

return FRAME;

function FRAME_ELIST

return FRAME;

function FRAME_ELIST (

EVENTS             : EVENT_LIST )

return FRAME;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PROCEDURE -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This subprogram performs signal assignments to the CONNECT parameter

-- based on the PERIOD and EVENTS parameters.  If the CODES parameter is

-- not null, then the character codes are applied to CONNECT as well.

-- The dimensions of CONNECT, EVENTS, and CODES (if present) must be

-- identical.  The subprogram waits for PERIOD.NOMINAL before returning

-- to the caller.  Event times must be non-decreasing.   Application of

-- the EVENTS parameter stops for a given signal when the tagged event

-- time is not specified (the flag is set to FALSE).

--

procedure APPLY (

signal CONNECT      : inout WAVES_PORT_LIST;

PERIOD              : in EVENT_TIME;

EVENTS              : in FRAME_SET;

CODES               : in STRING := "");

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PROCEDURE -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This subprogram performs signal assignments to the CONNECT parameter

-- based on the TAG_LABEL parameter.  The string in this parameter is

-- sent through the CONNECT parameter to the outside simulation

-- environment, and is used to pass messages (labels), mostly for debug

-- purposes.

--

procedure TAG (

signal CONNECT     : inout WAVES_PORT_LIST;

TAG_LABEL          : in STRING );

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTION -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This function returns the logical AND of the match values for every

-- element in the CONNECT parameter.

--

function MATCH (

CONNECT            : WAVES_PORT_LIST )

return BOOLEAN;

end WAVES_INTERFACE;

**************************************************************************************************

--

-- Copyright 1989 by ---

--

-- This code is distributed for the purposes of evaluating the

-- Waveform And Vector Exchange Specification (WAVES) proposal

-- presented to the IEEE by the WAVES Analysis Group.  This code

-- may not be used for commercial purposes and may not be

-- redistributed without permission of the Chairman of the WAVES

-- Analysis Group, Mr Robert Hillman.

--

-- Address comments or questions to

--    Robert Hillman                   Lee Shombert

--    RADC/RBRP                        Harris Corporation

--    Griffis AFB, NY                  PO Box 94000  MS 16/4010

--    (315) 330-2241                   Melbourne, FL 32902

--                                     (407) 727-6040

--

--

-- This package depends on WAVES_LIBRARY.WAVES_PORT

--

package body WAVES_INTERFACE is

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- These two functions return the minimum of the inputs and the maximum

-- of the inputs respectively.  They are just service routines for other

-- procedures in this package.

--

function MIN(A, B : INTEGER) return INTEGER

is

begin

if A <= B then return A; else return B; end if;

end MIN;

function MAX(A, B : INTEGER) return INTEGER

is

begin

if A >= B then return A; else return B; end if;

end MAX;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following functions are used by to create and manipulate frames,

-- frame sets, and frame set arrays.  The first set of functions return

-- the incoming object with it last index range adjusted to be (1 to L),

-- truncating or padding with unspecified tagged events, as appropriate.

-- The second set of operations are array concatenation, treating a

-- frame set as an array of frames and a frame set array as an array of

-- frame sets.

--

function PROMOTE (

DATA               : FRAME_1 )

return FRAME_2

is

variable TEMP_2      : FRAME_2 (1 to 1, DATA'RANGE(1));

begin

for I in DATA'RANGE(1) loop

TEMP_2(1, I) := DATA(I);

end loop;

return TEMP_2;

end PROMOTE;

function PROMOTE (

DATA               : FRAME_2 )

return FRAME_3

is

variable TEMP_3      : FRAME_3 (1 to 1, DATA'RANGE(1), DATA'RANGE(2));

begin

for I in DATA'RANGE(1) loop

for J in DATA'RANGE(2) loop

TEMP_3(1, I, J) := DATA(I, J);

end loop;

end loop;

return TEMP_3;

end PROMOTE;

function RESIZE (

DATA               : FRAME_1;

LENGTH_1           : POSITIVE )

return FRAME_1

is

variable TEMP_1      : FRAME_1 (1 to LENGTH_1);

begin

for I in DATA'RANGE(1) loop

TEMP_1(I).VALUE := 0;

end loop;

for I in 1 to MIN(LENGTH_1, DATA'LENGTH(1)) loop

TEMP_1(I) := DATA(I);

end loop;

return TEMP_1;

end RESIZE;

function RESIZE (

DATA               : FRAME_2;

LENGTH_2           : POSITIVE;

LENGTH_1           : POSITIVE )

return FRAME_2

is

variable TEMP_2      : FRAME_2 (1 to LENGTH_2, 1 to LENGTH_1);

begin

for I in TEMP_2'RANGE(1) loop

for J in TEMP_2'RANGE(2) loop

TEMP_2(I, J).VALUE := 0;

end loop;

end loop;

for I in 1 to MIN(LENGTH_2, DATA'LENGTH(1)) loop

for J in 1 to MIN(LENGTH_1, DATA'LENGTH(2)) loop

TEMP_2(I, J) := DATA(I, J);

end loop;

end loop;

return TEMP_2;

end RESIZE;

function RESIZE (

DATA               : FRAME_3;

LENGTH_3           : POSITIVE;

LENGTH_2           : POSITIVE;

LENGTH_1           : POSITIVE )

return FRAME_3

is

variable TEMP_3      : FRAME_3

(1 to LENGTH_3, 1 to LENGTH_2, 1 to LENGTH_1);

begin

for I in TEMP_3'RANGE(1) loop

for J in TEMP_3'RANGE(2) loop

for K in TEMP_3'RANGE(3) loop

TEMP_3(I, J, K).VALUE := 0;

end loop;

end loop;

end loop;

for I in 1 to MIN(LENGTH_3, DATA'LENGTH(1)) loop

for J in 1 to MIN(LENGTH_2, DATA'LENGTH(2)) loop

for K in 1 to MIN(LENGTH_1, DATA'LENGTH(3)) loop

TEMP_3(I, J, K) := DATA(I, J, K);

end loop;

end loop;

end loop;

return TEMP_3;

end RESIZE;

-- The following three functions trim unspecified tagged events from

-- the various frame types.

--

function TRIM (

DATA               : FRAME_1 )

return FRAME_1

is

variable LENGTH_1    : NATURAL := 1;

begin

for I in DATA'LENGTH(1) downto 1 loop

if DATA(I).VALUE > 0 then

LENGTH_1 := I;

exit;

end if;

end loop;

return RESIZE(DATA, LENGTH_1);

end TRIM;

function TRIM (

DATA               : FRAME_2 )

return FRAME_2

is

variable LENGTH_1    : NATURAL := 1;

variable LENGTH_2    : NATURAL := 1;

begin

for I in DATA'LENGTH(1) downto 1 loop

for J in DATA'LENGTH(2) downto 1 loop

if DATA(I, J).VALUE > 0 then

LENGTH_1 := I;

exit;

end if;

end loop;

end loop;

for J in DATA'LENGTH(2) downto 1 loop

for I in DATA'LENGTH(1) downto 1 loop

if DATA(I, J).VALUE > 0 then

LENGTH_2 := J;

exit;

end if;

end loop;

end loop;

return RESIZE(DATA, LENGTH_1, LENGTH_2);

end TRIM;

function TRIM (

DATA               : FRAME_3 )

return FRAME_3

is

variable LENGTH_1    : NATURAL := 1;

variable LENGTH_2    : NATURAL := 1;

variable LENGTH_3    : NATURAL := 1;

begin

for I in DATA'LENGTH(1) downto 1 loop

for J in DATA'LENGTH(2) downto 1 loop

for K in DATA'LENGTH(3) downto 1 loop

if DATA(I, J, K).VALUE > 0 then

LENGTH_1 := I;

exit;

end if;

end loop;

end loop;

end loop;

for J in DATA'LENGTH(2) downto 1 loop

for I in DATA'LENGTH(1) downto 1 loop

for K in DATA'LENGTH(3) downto 1 loop

if DATA(I, J, K).VALUE > 0 then

LENGTH_2 := I;

exit;

end if;

end loop;

end loop;

end loop;

for K in DATA'LENGTH(3) downto 1 loop

for I in DATA'LENGTH(1) downto 1 loop

for J in DATA'LENGTH(2) downto 1 loop

if DATA(I, J, K).VALUE > 0 then

LENGTH_3 := I;

exit;

end if;

end loop;

end loop;

end loop;

return RESIZE(DATA, LENGTH_1, LENGTH_2, LENGTH_3);

end TRIM;

-- The following functions concatenate two objects of the same

-- dimensionality, returning a similar object of the same

-- dimensionality.  The effect is similar to the concatentation of

-- one dimensional arrays.  Secondary array dimensions are adjusted

-- prior to concatenation.  Normally, one would trim these arrays prior

-- to concatenation to eliminate "null" rows of tagged events.

--

function CONCAT(A, B : FRAME_1) return FRAME_1

is

variable INDEX       : NATURAL := 0;

variable TEMP_1      : FRAME_1 (1 to A'LENGTH + B'LENGTH);

begin

for I in 1 to A'LENGTH(1) loop

TEMP_1(I) := A(I);

end loop;

for I in 1 to B'LENGTH(1) loop

TEMP_1(I + A'LENGTH(1)) := B(I);

end loop;

return TEMP_1;

end CONCAT;

function CONCAT(A, B : FRAME_2) return FRAME_2

is

variable LENGTH_1    : POSITIVE := MAX(A'LENGTH(2), B'LENGTH(2));

variable TEMP_2      : FRAME_2

(1 to A'LENGTH(1) + B'LENGTH(1), 1 to LENGTH_1);

begin

if A'LENGTH(2) /= B'LENGTH(2) then

return CONCAT(RESIZE(A,A'LENGTH(1), LENGTH_1), RESIZE(B,B'LENGTH(1), LENGTH_1));

else

for I in 1 to A'LENGTH(1) loop

for J in 1 to LENGTH_1 loop

TEMP_2(I, J) := A(I, J);

end loop;

end loop;

for I in 1 to B'LENGTH(1) loop

for J in 1 to LENGTH_1 loop

TEMP_2(I + A'LENGTH(1), J) := B(I, J);

end loop;

end loop;

return TEMP_2;

end if;

end CONCAT;

function CONCAT(A, B : FRAME_3) return FRAME_3

is

variable LENGTH_1    : POSITIVE := MAX(A'LENGTH(3), B'LENGTH(3));

variable LENGTH_2    : POSITIVE := MAX(A'LENGTH(2), B'LENGTH(2));

variable TEMP_3      : FRAME_3

(1 to A'LENGTH(1) + B'LENGTH(1), 1 to LENGTH_2, 1 to LENGTH_1);

begin

if A'LENGTH(2) /= B'LENGTH(2) or A'LENGTH(3) /= B'LENGTH(3) then

return CONCAT(RESIZE(A,A'LENGTH(1), LENGTH_2, LENGTH_1),

RESIZE(B,B'LENGTH(1), LENGTH_2, LENGTH_1));

else

for I in 1 to A'LENGTH(1) loop

for J in 1 to LENGTH_2 loop

for K in 1 to LENGTH_1 loop

TEMP_3(I, J, K) := A(I, J, K);

end loop;

end loop;

end loop;

for I in 1 to B'LENGTH(1) loop

for J in 1 to LENGTH_2 loop

for K in 1 to LENGTH_1 loop

TEMP_3(I + A'LENGTH(1), J, K) := B(I, J, K);

end loop;

end loop;

end loop;

return TEMP_3;

end if;

end CONCAT;

-- The following functions allow a WAVES dataset to create a frame set

-- from a set of frames using the concatenation operator.  Input

-- parameters are not trimmed, to allow frame sets to contain empty

-- frames.

--

function "+"(A : FRAME_1;  B : FRAME_1) return FRAME_2

is

begin

return CONCAT(PROMOTE(A), PROMOTE(B));

end "+";

function "+"(A : FRAME_2;  B : FRAME_1) return FRAME_2

is

begin

return CONCAT(A, PROMOTE(B));

end "+";

function "+"(A : FRAME_1;  B : FRAME_2) return FRAME_2

is

begin

return CONCAT(PROMOTE(A), B);

end "+";

function "+"(A : FRAME_2;  B : FRAME_2) return FRAME_2

is

begin

return CONCAT(A, B);

end "+";

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following functions convert a single event or a single event list

-- into a frame (a list of tagged events).  The empty case returns a

-- frame of length one with the tagged event marked unspecified.

--

function FRAME_EVENT

return FRAME

is

variable TEMP_1      : FRAME_1 (1 to 1);

begin

TEMP_1(1).VALUE := 0;

return TEMP_1;

end FRAME_EVENT;

function FRAME_EVENT (

EVENTS             : EVENT )

return FRAME

is

variable TEMP_1      : FRAME_1 (1 to 1);

begin

TEMP_1(1).VALUE := EVENTS.VALUE;

TEMP_1(1).TIME  := EVENTS.TIME;

return TEMP_1;

end FRAME_EVENT;

function FRAME_ELIST

return FRAME

is

variable TEMP_1      : FRAME_1 (1 to 1);

begin

TEMP_1(1).VALUE := 0;

return TEMP_1;

end FRAME_ELIST;

function FRAME_ELIST (

EVENTS             : EVENT_LIST )

return FRAME

is

variable TEMP_1      : FRAME_1 (1 to EVENTS'LENGTH);

begin

for I in 1 to EVENTS'LENGTH loop

TEMP_1(I).VALUE := EVENTS(I).VALUE;

TEMP_1(I).TIME  := EVENTS(I).TIME;

end loop;

return TEMP_1;

end FRAME_ELIST;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PROCEDURE -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This subprogram performs signal assignments to the CONNECT parameter

-- based on the PERIOD and EVENTS parameters.  If the CODES parameter is

-- not null, then the character codes are applied to CONNECT as well.

-- The dimensions of CONNECT, EVENTS, and CODES (if present) must be

-- identical.  The subprogram waits for PERIOD.NOMINAL before returning

-- to the caller.

--

procedure APPLY (

signal CONNECT     : inout WAVES_PORT_LIST;

PERIOD             : in EVENT_TIME;

EVENTS             : in FRAME_SET;

CODES              : in STRING := "")

is

begin

assert CONNECT'LENGTH = EVENTS'LENGTH(1);

assert CODES'LENGTH = 0 or

CODES'LENGTH = CONNECT'LENGTH;

for PIN in CONNECT'RANGE loop

for E in EVENTS'RANGE(2) loop

exit when EVENTS(PIN,E).VALUE <= 0;

CONNECT(PIN).DATA <= transport

EVENTS(PIN, E).VALUE after EVENTS(PIN, E).TIME.NOMINAL;

end loop;

if CODES'LENGTH > 0 then

CONNECT(PIN).CODE <= transport CODES(PIN);

else

CONNECT(PIN).CODE <= transport NUL;

end if;

end loop;

wait for PERIOD.NOMINAL;

end APPLY;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PROCEDURE -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This subprogram performs signal assignments to the CONNECT parameter

-- based on the TAG_LABEL parameter.  The string in this parameter is

-- sent through the CONNECT parameter to the outside simulation

-- environment, and is used to pass messages (labels), mostly for debug

-- purposes.

--

procedure TAG (

signal CONNECT     : inout WAVES_PORT_LIST;

TAG_LABEL          : in STRING )

is

begin

for I in TAG_LABEL'RANGE loop

CONNECT(CONNECT'LEFT).MESSAGE <= transport TAG_LABEL(I);

wait for 0 ns;

end loop;

end TAG;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTION -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This function returns the logical AND of the match values for every

-- element in the CONNECT parameter.

--

function MATCH (

CONNECT            : WAVES_PORT_LIST )

return BOOLEAN

is

variable FLAG        : BOOLEAN := TRUE;

begin

for I in CONNECT'RANGE loop

FLAG := FLAG and CONNECT(I).MATCH;

end loop;

return FLAG;

end MATCH;

end WAVES_INTERFACE;

**************************************************************************************************

--

-- Copyright 1989 by ---

--

-- This code is distributed for the purposes of evaluating the

-- Waveform And Vector Exchange Specification (WAVES) proposal

-- presented to the IEEE by the WAVES Analysis Group.  This code

-- may not be used for commercial purposes and may not be

-- redistributed without permission of the Chairman of the WAVES

-- Analysis Group, Mr Robert Hillman.

--

-- Address comments or questions to

--    Robert Hillman                   Lee Shombert

--    RADC/RBRP                        Harris Corporation

--    Griffis AFB, NY                  PO Box 94000  MS 16/4010

--    (315) 330-2241                   Melbourne, FL 32902

--                                     (407) 727-6040

--

--

-- This is the standard package used by all WAVES datasets

--

library WAVES_LIBRARY;

use WAVES_LIBRARY.WAVES_STANDARD.all;

package WAVES_PORT is

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- SYSTEM -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This type is the most likely to change.  In this package, DATA is

-- driven by the WAVES dataset and represents an EVENT_VALUE on a node.

-- The actual EVENT_VALUE is selected from the VALUE_DICTIONARY used

-- by the WAVES dataset.  CODE is a character that may be used for

-- reporting purposes.  MATCH is a boolean coming into the WAVES dataset

-- and represents the result of comparing an OUTPUT or COMPOUND

-- EVENT_VALUEs with the outputs of some other component in the

-- simulation environment.  The MESSAGE field receives "tags" from the

-- waveform, indicating the occurence of labels.

--

type WAVES_PORT is record

DATA                  : NATURAL;

CODE                  : CHARACTER;

MATCH                 : BOOLEAN;

MESSAGE               : CHARACTER;

end record;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This is the basic interface type (based on the local definition of

-- WAVES_PORT).

--

type WAVES_PORT_LIST is array (POSITIVE range <>) of WAVES_PORT;

end WAVES_PORT;

**************************************************************************************************

--

-- Copyright 1989 by ---

--

-- This code is distributed for the purposes of evaluating the

-- Waveform And Vector Exchange Specification (WAVES) proposal

-- presented to the IEEE by the WAVES Analysis Group.  This code

-- may not be used for commercial purposes and may not be

-- redistributed without permission of the Chairman of the WAVES

-- Analysis Group, Mr Robert Hillman.

--

-- Address comments or questions to

--    Robert Hillman                   Lee Shombert

--    RADC/RBRP                        Harris Corporation

--    Griffis AFB, NY                  PO Box 94000  MS 16/4010

--    (315) 330-2241                   Melbourne, FL 32902

--                                     (407) 727-6040

--

--

-- This is the standard package used by all WAVES datasets

--

library WAVES_LIBRARY;

use WAVES_LIBRARY.WAVES_STANDARD.all;

package body WAVES_PORT is

end WAVES_PORT;

**************************************************************************************************

--

-- Copyright 1989 by ---

--

-- This code is distributed for the purposes of evaluating the

-- Waveform And Vector Exchange Specification (WAVES) proposal

-- presented to the IEEE by the WAVES Analysis Group.  This code

-- may not be used for commercial purposes and may not be

-- redistributed without permission of the Chairman of the WAVES

-- Analysis Group, Mr Robert Hillman.

--

-- Address comments or questions to

--    Robert Hillman                   Lee Shombert

--    RADC/RBRP                        Harris Corporation

--    Griffis AFB, NY                  PO Box 94000  MS 16/4010

--    (315) 330-2241                   Melbourne, FL 32902

--                                     (407) 727-6040

--

--

--

--

library WAVES_LIBRARY;

use STD.TEXTIO.all;

use WAVES_LIBRARY.WAVES_STANDARD.all;

use WAVES_LIBRARY.WAVES_PORT.all;

use WAVES_LIBRARY.WAVES_INTERFACE.all;

use WORK.WAVES_GENERATOR.all;

package WAVES_OBJECTS is

--

-- START OF WAVES_OBJECTS PACKAGE SPECIFICATION

--

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- These definitions create ranges used in dimensioning arrays.  They

-- depend on values defined in the WAVES_GENERATOR specification.  The

-- associated functions return the index of a TEST_PINS element in any

-- array dimensioned by TEST_PIN_RANGE, and the index of a character

-- code in the PIN_CODES string (-1 if not present).

--

subtype TEST_PIN_RANGE is INTEGER range

TEST_PINS'POS(TEST_PINS'LEFT) + 1 to

TEST_PINS'POS(TEST_PINS'RIGHT) + 1;

subtype PIN_CODE_RANGE is INTEGER range

PIN_CODES'LEFT to PIN_CODES'RIGHT;

function INDEX (

VALUE              : TEST_PINS )

return INTEGER;

function INDEX (

VALUE              : CHARACTER )

return INTEGER;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following definitions build up to a WAVE SLICE.

--

-- A FRAME SET ARRAY POINTER points to a FRAME SET ARRAY

--

-- A TIME_DATA_BLOCK contains a FRAME_SET_ARRAY_POINTER and an EVENT_TIME.

-- When associated with a WAVE_SLICE, the EVENT_TIME provides tolerances

-- for the PERIOD.

--

-- A TIME DATA is a pointer to a TIME_DATA_BLOCK.  A pointer is used to

-- save space, since only a few TIME_DATA_BLOCKs are typically defined,

-- but these are referenced in many places.

--

-- A WAVE_TIMING contains all timing information needed for a

-- WAVE_SLICE.  It contains a period which specifies the duration of a

-- slice.  It contains a time data object which specifies the frame sets

-- associated with all the pins in the dataset for that slice.  WAVE_

-- TIMING is an object similar in concept to time sets in AMTE.

--

-- A WAVE_SLICE is a record that contains all information needed to

-- generate a waveform across a set of pins.  The CODES field

-- carries a character for every pin.  It is an error if any character

-- does not appear in the package specification constant PIN_CODES.

-- The TIMING field carries all the timing for the slice.

--

type FRAME_SET_ARRAY_POINTER is access FRAME_SET_ARRAY;

type TIME_DATA_BLOCK is record

FRAME_SET_DATA        : FRAME_SET_ARRAY_POINTER;

PERIOD_TOLERANCE      : EVENT_TIME;

end record;

type TIME_DATA is access TIME_DATA_BLOCK;

type WAVE_TIMING is record

PERIOD                : TIME;

TIMING                : TIME_DATA;

end record;

type WAVE_SLICE is record

CODES                 : STRING (TEST_PIN_RANGE);

TIMING                : WAVE_TIMING;

end record;

type WAVE_SLICE_LIST is array (POSITIVE range <>) of WAVE_SLICE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following definitions allow unconstrained arrays of WAVES

-- objects.

--

type TIME_LIST is array (POSITIVE range <>) of TIME;

type EVENT_TIME_LIST is array (POSITIVE range <>) of EVENT_TIME;

type TIME_DATA_LIST is array (POSITIVE range <>) of TIME_DATA;

type WAVE_TIMING_LIST is array (POSITIVE range <>) of WAVE_TIMING;

type TEST_PINS_LIST is array (POSITIVE range <>) of TEST_PINS;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following definitions provide support for pinsets.  A PINSET is

-- an array of booleans, one for each test pin.  Two defered constants

-- are defined, ALL PINS and NO PINS.

--

-- A TEST_PINS_LIST is an array of TEST_PINS.  Objects of this type are

-- used as parameters to functions that create pinsets.

--

type PINSET is array (TEST_PIN_RANGE) of BOOLEAN;

constant ALL_PINS       : PINSET;

constant NO_PINS        : PINSET;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- Two functions are provided to create a pinset.  The first takes a

-- single test pin and returns a pinset containing only that test pin.

-- The second contains a  list of test pins and returns a pinset

-- containing all of the listed test pins.  These are overloaded to

-- reduce the name space.

--

-- Pinsets may be added and subtracted using the predefined VHDL logical

-- operators, which operate on boolean arrays.  These are:

--   function  or ( A, B : PINSET ) return PINSET;

--   function and ( A, B : PINSET ) return PINSET;

--   function not ( A, B : PINSET ) return PINSET;

--

function NEW_PINSET (

PIN                 : in TEST_PINS ) return PINSET;

function NEW_PINSET (

PINS                : in TEST_PINS_LIST ) return PINSET;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The first procedure applies a single wave slice to the outputs of the

-- waveform generator package.  The MATCH functions scan all or some of

-- the waveform generator signals returning TRUE if every signal matches.

-- The TAG procedures apply the string parameter to the output signals,

-- in effect labeling (tagging) the next call to APPLY.

--

procedure APPLY_SLICE (

signal CONNECT      : inout WAVES_PORT_LIST (TEST_PIN_RANGE);

DATA                : in WAVE_SLICE );

procedure APPLY_LIST (

signal CONNECT      : inout WAVES_PORT_LIST (TEST_PIN_RANGE);

DATA                : in WAVE_SLICE_LIST );

function MATCH (

CONNECT             : WAVES_PORT_LIST (TEST_PIN_RANGE);

PINS                : PINSET )

return BOOLEAN;

function MATCH (

CONNECT             : WAVES_PORT_LIST (TEST_PIN_RANGE);

PINS                : TEST_PINS )

return BOOLEAN;

procedure TAG (

signal CONNECT     : inout WAVES_PORT_LIST (TEST_PIN_RANGE);

TAG_LABEL          : STRING;

PINS               : PINSET );

procedure TAG (

signal CONNECT     : inout WAVES_PORT_LIST (TEST_PIN_RANGE);

TAG_LABEL          : STRING;

PINS               : TEST_PINS );

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following functions are used to create the objects required for a

-- TIME_DATA object.

--

function NEW_FRAME_SET_ARRAY (

PINS               : TEST_PINS;

FRAMES             : FRAME_SET )

return FRAME_SET_ARRAY;

function NEW_FRAME_SET_ARRAY (

PINS               : PINSET;

FRAMES             : FRAME_SET )

return FRAME_SET_ARRAY;

function ADD_FRAME_SET_ARRAY (

A                  : FRAME_SET_ARRAY;

B                  : FRAME_SET_ARRAY )

return FRAME_SET_ARRAY;

function NEW_TIME_DATA (

FRAMES             : FRAME_SET_ARRAY;

PERIOD_TOLERANCE   : EVENT_TIME := ( 0 ns, FALSE, 0 ns, 0 ns) )

return TIME_DATA;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following function converts an integer into a binary code.  The

-- length of the result is specified in the parameter WIDTH and the

-- character codes to be used for binary 0 and binary 1 are in the

-- parameter KEY.

--

function BIT_STRING (

VALUE              : INTEGER;

KEY                : STRING (1 to 2);

WIDTH              : NATURAL )

return STRING;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following definitions support the fixed file format of Level 1

-- WAVES.  The FILE INPUT DATA record stores information most recently

-- read by the READ WAVE SLICE procedure.  A flag, PERIOD NOT INDEX, is

-- set to indicate whether the file contained a time or an integer most

-- recently.  The flag END_OF_FILE is set if an EOF is encountered while

-- scanning for the current wave slice.  END_OF_FILE is always FALSE if

-- a wave slice is successfully (no errors) read.

--

--

--  White space is permitted between any of the tokens in the following

--  syntax.

--

--  waves_file ::=

--      { slice } [ '#' ].

--  slice ::=

--      [ quoted_string ] { adjusted_string } { timing } ';'.

--  adjusted_string ::=

--      ( '=' integer | '&' ) quoted_string.

--  timing ::=

--      ':' ( integer | time ).

--

--  White space is disallowed in the following syntax except where

--  specifically permitted.

--

--  quoted_string ::=

--      ( '"' character | alphanum ) non_blank_string.

--  integer ::=

--      vhdl_integer.

--  time ::=

--      vhdl_time.

--

type FILE_INPUT_DATA is record

CODES                 : STRING (TEST_PIN_RANGE);

PERIOD_SET            : BOOLEAN;

PERIOD                : TIME;

INDEX_SET             : BOOLEAN;

INDEX                 : INTEGER;

END_OF_FILE           : BOOLEAN;

end record;

procedure READ_WAVE_SLICE (

variable IFILE      : in TEXT;

ACCUMULATOR         : inout FILE_INPUT_DATA );

--

-- END OF WAVES_OBJECTS PACKAGE SPECIFICATION

--

end WAVES_OBJECTS;

**************************************************************************************************

--

-- Copyright 1989 by ---

--

-- This code is distributed for the purposes of evaluating the

-- Waveform And Vector Exchange Specification (WAVES) proposal

-- presented to the IEEE by the WAVES Analysis Group.  This code

-- may not be used for commercial purposes and may not be

-- redistributed without permission of the Chairman of the WAVES

-- Analysis Group, Mr Robert Hillman.

--

-- Address comments or questions to

--    Robert Hillman                   Lee Shombert

--    RADC/RBRP                        Harris Corporation

--    Griffis AFB, NY                  PO Box 94000  MS 16/4010

--    (315) 330-2241                   Melbourne, FL 32902

--                                     (407) 727-6040

--

--

--

--

package body WAVES_OBJECTS is

--

-- START OF WAVES_OBJECTS PACKAGE BODY

--

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- Two useful functions.

--

function MAX(A, B: INTEGER) return INTEGER is

begin

if A >= B then return A; else return B; end if;

end MAX;

function MIN(A, B: INTEGER) return INTEGER is

begin

if A <= B then return A; else return B; end if;

end MIN;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- INTERNAL =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- An internal function to translate from a character to an index within

-- PIN_CODE_RANGE.  A negative number is returned if the input character

-- is not within the string constant PIN_CODES.

--

function INDEX (

VALUE              : CHARACTER )

return INTEGER

is

begin

for C in PIN_CODES'LEFT to PIN_CODES'RIGHT loop

if PIN_CODES(C) = VALUE then

return C;

end if;

end loop;

return -1;

end INDEX;

function INDEX (

VALUE              : TEST_PINS )

return INTEGER

is

begin

return TEST_PINS'POS(VALUE) + 1;

end INDEX;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This procedure applies a single wave slice to the outputs of the

-- waveform generator package.

--

procedure APPLY_SLICE (

signal CONNECT      : inout WAVES_PORT_LIST (TEST_PIN_RANGE);

DATA                : in WAVE_SLICE )

is

variable C            : CHARACTER;

variable D            : TIME_DATA := DATA.TIMING.TIMING;

variable E            : EVENT_TIME;

variable T            : STRING (TEST_PIN_RANGE) := DATA.CODES;

begin

if D = null then

assert FALSE

report "Malformed wave slice - missing time data"

severity ERROR;

else

for PIN in TEST_PIN_RANGE loop

C := T(PIN);

if INDEX(C) = -1 then

assert FALSE

report "Illegal character code in wave slice"

severity ERROR;

else

for I in 1 to D.FRAME_SET_DATA.all'LENGTH(3) loop

exit when D.FRAME_SET_DATA(PIN, INDEX(C), I).VALUE <= 0;

CONNECT(PIN).DATA <= transport

D.FRAME_SET_DATA(PIN, INDEX(C), I).VALUE

after D.FRAME_SET_DATA(PIN, INDEX(C), I).TIME.NOMINAL;

end loop;

CONNECT(PIN).CODE <= transport C;

end if;

end loop;

E := D.PERIOD_TOLERANCE;

E.NOMINAL := DATA.TIMING.PERIOD;

wait for E.NOMINAL;

end if;

end APPLY_SLICE;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This procedure applies a list of wave slices to the outputs of the

-- waveform generator package.

--

procedure APPLY_LIST (

signal CONNECT      : inout WAVES_PORT_LIST (TEST_PIN_RANGE);

DATA                : in WAVE_SLICE_LIST )

is

variable X            : WAVE_SLICE;

begin

for W in DATA'RANGE loop

X := DATA(W);

APPLY_SLICE(CONNECT, X);

end loop;

end APPLY_LIST;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This function returns the AND of the MATCH signals of the specified

-- pins.

--

function MATCH (

CONNECT             : WAVES_PORT_LIST (TEST_PIN_RANGE);

PINS                : PINSET )

return BOOLEAN

is

variable FLAG        : BOOLEAN := TRUE;

begin

for PIN in CONNECT'RANGE loop

if PINS(PIN) then

FLAG := FLAG and CONNECT(PIN).MATCH;

end if;

end loop;

return FLAG;

end MATCH;

function MATCH (

CONNECT             : WAVES_PORT_LIST (TEST_PIN_RANGE);

PINS                : TEST_PINS )

return BOOLEAN

is

begin

return CONNECT(INDEX(PINS)).MATCH;

end MATCH;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- This procedure applies a WAVES label to the output ports of the

-- waveform generator.

--

procedure TAG (

signal CONNECT     : inout WAVES_PORT_LIST (TEST_PIN_RANGE);

TAG_LABEL          : STRING;

PINS               : PINSET )

is

begin

for P in PINS'RANGE loop

if PINS(P) then

for I in TAG_LABEL'range loop

CONNECT(I).MESSAGE <= transport TAG_LABEL(I);

wait for 0 ns;

end loop;

end if;

end loop;

end TAG;

procedure TAG (

signal CONNECT     : inout WAVES_PORT_LIST (TEST_PIN_RANGE);

TAG_LABEL          : STRING;

PINS               : TEST_PINS )

is

begin

for I in TAG_LABEL'RANGE loop

CONNECT(INDEX(PINS)).MESSAGE <= transport TAG_LABEL(I);

wait for 0 ns;

end loop;

end TAG;

--

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- Two defered constants.

--

constant ALL_PINS       : PINSET := (others => TRUE);

constant NO_PINS        : PINSET := (others => FALSE);

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- Two functions are provided to create a pinset.  The first takes a

-- single test pin and returns a pinset containing only that test pin.

-- The second contains a  list of test pins and returns a pinset

-- containing all of the listed test pins.  These are overloaded to

-- reduce the name space.

--

-- Pinsets may be added and subtracted using the predefined VHDL logical

-- operators, which operate on boolean arrays.  These are:

--   function  or ( A, B : PINSET ) return PINSET;

--   function and ( A, B : PINSET ) return PINSET;

--   function not ( A, B : PINSET ) return PINSET;

--

function NEW_PINSET (

PIN                 : in TEST_PINS )

return PINSET

is

variable X            : PINSET := (others => FALSE);

begin

X(INDEX(PIN)) := TRUE;

return X;

end NEW_PINSET;

function NEW_PINSET (

PINS                : in TEST_PINS_LIST )

return PINSET

is

variable X            : PINSET := (others => FALSE);

begin

for PIN in PINS'RANGE loop

X(INDEX(PINS(PIN))) := TRUE;

end loop;

return X;

end NEW_PINSET;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following functions are used to create the objects required for a

-- TIME_DATA object.

--

function NEW_FRAME_SET (

CODES              : STRING;

EVENTS             : FRAME )

return FRAME_SET

is

variable X           : FRAME_SET (PIN_CODE_RANGE, 1 to EVENTS'LENGTH);

begin

for I in CODES'RANGE loop

if INDEX (CODES(I)) /= -1 then

for J in X'RANGE(2) loop

X(INDEX(CODES(I)), J) := EVENTS(J);

end loop;

else

assert FALSE

report "Error in new frame set pin code select"

severity ERROR;

end if;

end loop;

return X;

end NEW_FRAME_SET;

function ADD_FRAME_SET (

A                  : FRAME_SET;

B                  : FRAME_SET )

return FRAME_SET

is

variable F           : BOOLEAN;

variable X           : FRAME_SET

(PIN_CODE_RANGE, 1 to MAX(A'LENGTH(2), B'LENGTH(2)));

begin

assert A'LENGTH(1) = PIN_CODE_RANGE'RIGHT and

B'LENGTH(1) = PIN_CODE_RANGE'RIGHT

report "Bad frame set size in ADD FRAME SET"

severity ERROR;

for I in PIN_CODE_RANGE loop

F := FALSE;

for J in 1 to A'LENGTH(2) loop

X(I,J) := A(I,J);

F := F or (X(I, J).VALUE > 0);

end loop;

if not F then

for J in 1 to B'LENGTH(2) loop

X(I,J) := B(I,J);

end loop;

end if;

end loop;

return X;

end ADD_FRAME_SET;

function NEW_FRAME_SET_ARRAY (

PINS               : TEST_PINS;

FRAMES             : FRAME_SET )

return FRAME_SET_ARRAY

is

variable X           : FRAME_SET_ARRAY

(TEST_PIN_RANGE, PIN_CODE_RANGE, 1 to FRAMES'LENGTH(2));

begin

assert FRAMES'LENGTH(1) = PIN_CODE_RANGE'RIGHT

report "Bad frame set size in NEW FRAME SET ARRAY"

severity ERROR;

for I in X'RANGE(2) loop

for J in X'RANGE(3) loop

X(INDEX(PINS), I, J) := FRAMES(I, J);

end loop;

end loop;

return X;

end NEW_FRAME_SET_ARRAY;

function NEW_FRAME_SET_ARRAY (

PINS               : PINSET;

FRAMES             : FRAME_SET )

return FRAME_SET_ARRAY

is

variable X           : FRAME_SET_ARRAY

(TEST_PIN_RANGE, PIN_CODE_RANGE, 1 to FRAMES'LENGTH(2));

begin

assert FRAMES'LENGTH(1) = PIN_CODE_RANGE'RIGHT

report "Bad frame set size in NEW FRAME SET ARRAY"

severity ERROR;

for PIN in TEST_PIN_RANGE loop

if PINS(PIN) then

for I in X'RANGE(2) loop

for J in X'RANGE(3) loop

X(PIN, I, J) := FRAMES(I, J);

end loop;

end loop;

end if;

end loop;

return X;

end NEW_FRAME_SET_ARRAY;

function ADD_FRAME_SET_ARRAY (

A                  : FRAME_SET_ARRAY;

B                  : FRAME_SET_ARRAY )

return FRAME_SET_ARRAY

is

variable F           : BOOLEAN;

variable X           : FRAME_SET_ARRAY

(TEST_PIN_RANGE, PIN_CODE_RANGE, 1 to MAX(A'LENGTH(3), B'LENGTH(3)));

begin

assert A'LENGTH(1) = TEST_PIN_RANGE'RIGHT and

B'LENGTH(1) = TEST_PIN_RANGE'RIGHT and

A'LENGTH(2) = PIN_CODE_RANGE'RIGHT and

A'LENGTH(2) = PIN_CODE_RANGE'RIGHT

report "Bad frame set size in ADD FRAME SET ARRAY"

severity ERROR;

for PIN in TEST_PIN_RANGE loop

F := FALSE;

for I in X'RANGE(2) loop

for J in X'RANGE(3) loop

X(PIN, I, J) := A(PIN, I, J);

F := F or (X(PIN, I, J).VALUE > 0);

end loop;

end loop;

if not F then

for I in X'RANGE(2) loop

for J in X'RANGE(3) loop

X(PIN, I, J) := B(PIN, I, J);

end loop;

end loop;

end if;

end loop;

return X;

end ADD_FRAME_SET_ARRAY;

function NEW_TIME_DATA (

FRAMES             : FRAME_SET_ARRAY;

PERIOD_TOLERANCE   : EVENT_TIME := ( 0 ns, FALSE, 0 ns, 0 ns) )

return TIME_DATA

is

begin

return new TIME_DATA_BLOCK'(

new FRAME_SET_ARRAY'(FRAMES),

PERIOD_TOLERANCE);

end NEW_TIME_DATA;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=- FUNCTIONS -=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following function converts an integer into a binary code.  The

-- length of the result is specified in the parameter WIDTH and the

-- character codes to be used for binary 0 and binary 1 are in the

-- parameter KEY.

--

function BIT_STRING (

VALUE              : INTEGER;

KEY                : STRING (1 to 2);

WIDTH              : NATURAL )

return STRING

is

variable X           : STRING (1 to WIDTH);

begin

for I in 1 to WIDTH loop

end loop;

return X;

end BIT_STRING;

-- -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=- PUBLIC -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

--

-- The following definitions support the fixed file format of Level 1

-- WAVES.  The FILE INPUT DATA record stores information most recently

-- read by the READ WAVE SLICE procedure.  A flag, PERIOD NOT INDEX, is

-- set to indicate whether the file contained a time or an integer most

-- recently.  The flag END_OF_FILE is set if an EOF is encountered while

-- scanning for the current wave slice.  END_OF_FILE is always FALSE if

-- a wave slice is successfully (no errors) read.

--

procedure READ_WAVE_SLICE (

variable IFILE      : in TEXT;

ACCUMULATOR         : inout FILE_INPUT_DATA )

is

variable CHAR         : CHARACTER;

variable EOL          : BOOLEAN;

variable END_OF_SLICE : BOOLEAN;

variable GOOD         : BOOLEAN;

variable ILINE        : LINE;

variable INDEX        : NATURAL;

-- Return TRUE if the input character is white space.

--

function IS_WHITE (

C                : CHARACTER )

return BOOLEAN

is

begin

return (C = ' ') or (C = HT);

end IS_WHITE;

-- Ensure that a non-white character remains on the current line,

-- reading new lines as needed.  The output flag is set to TRUE if the

-- procedure was unsuccessful (indicating an unexpected EOF).

--

procedure SKIP_WHITE (

END_OF_FILE      : out BOOLEAN )

is

variable EOL       : BOOLEAN;

variable TRASH     : CHARACTER;

begin

END_OF_FILE := FALSE;

loop

while ILINE.all'LENGTH > 0 and IS_WHITE(ILINE(ILINE.all'LEFT)) loop

READ(ILINE, TRASH);

end loop;

exit when ILINE.all'LENGTH > 0;

END_OF_FILE := FALSE;

READLINE(IFILE, ILINE);

end loop;

end SKIP_WHITE;

--

-- Read the next non-white character from the input line, reading new

-- lines as necessary.

--

procedure READ_CHAR

is

begin

SKIP_WHITE(ACCUMULATOR.END_OF_FILE);

READ(ILINE, CHAR);

end READ_CHAR;

-- Read an index from the file.

--

procedure UPDATE_INDEX

is

begin

SKIP_WHITE(ACCUMULATOR.END_OF_FILE);

READ(ILINE, INDEX, GOOD);

assert GOOD

report "Error reading index from file format"

severity ERROR;

if GOOD then

assert INDEX >= TEST_PIN_RANGE'LEFT and

INDEX <= TEST_PIN_RANGE'RIGHT

report "Error - index out of bounds in file format"

severity ERROR;

end if;

end UPDATE_INDEX;

-- Read either a time or an integer for timing data.  Look for a

-- closing semicolon.

--

procedure GET_TIMING

is

begin

SKIP_WHITE(ACCUMULATOR.END_OF_FILE);

READ(ILINE, ACCUMULATOR.PERIOD, GOOD);

if GOOD then

ACCUMULATOR.PERIOD_SET := TRUE;

else

READ(ILINE, ACCUMULATOR.INDEX, GOOD);

if GOOD then

ACCUMULATOR.INDEX_SET := TRUE;

else

assert FALSE

report "Error in timing in file format"

severity ERROR;

end if;

end if;

end GET_TIMING;

--

-- Read a quoted string from the external file.  If the first

-- character is a double quote, unconditionally accept the following

-- character.  Continue reading until white space is seen.

--

procedure GET_QUOTED_STRING (

USE_LAST_CHAR    : BOOLEAN := FALSE )

is

begin

if not USE_LAST_CHAR then

READ_CHAR;

if ACCUMULATOR.END_OF_FILE then

return;

end if;

end if;

if CHAR = '"' then

READ(ILINE, CHAR);

end if;

ACCUMULATOR.CODES(INDEX) := CHAR;

INDEX := INDEX + 1;

loop

ENDLINE(ILINE, EOL);

exit when EOL;

READ(ILINE, CHAR);

exit when IS_WHITE(CHAR);

assert INDEX >= TEST_PIN_RANGE'LEFT and

INDEX <= TEST_PIN_RANGE'RIGHT

report "Error - index out of bounds in file format"

severity ERROR;

ACCUMULATOR.CODES(INDEX) := CHAR;

INDEX := INDEX + 1;

end loop;

end GET_QUOTED_STRING;

--

-- Determine type of next token.  This involves reading the next

-- character and casing off of it.  Special characters are the equal

-- sign, indicating that an index is coming, an ampersand, indicating

-- concatenation, a colon, indicating that a timing is coming, and a

-- semicolon, indicating the end of the slice.  For the first field

-- only, a quoted string is legal.  A pound sign indicates and end of

-- file, but is also only legal as the first field.

--

procedure READ_NEXT_FIELD (

FIRST            : in BOOLEAN := FALSE)

is

begin

READ_CHAR;

case CHAR is

when '=' =>

UPDATE_INDEX;

GET_QUOTED_STRING;

when '&' =>

GET_QUOTED_STRING;

when ':' =>

GET_TIMING;

when ';' =>

END_OF_SLICE := TRUE;

when '#' =>

ACCUMULATOR.END_OF_FILE := TRUE;

assert FIRST

report "Error in file format - unexpected EOF command"

severity ERROR;

when others =>

if FIRST then

GET_QUOTED_STRING(TRUE);

else

assert FALSE

report "Error in file format - expected one of =&:;"

severity ERROR;

end if;

end case;

end READ_NEXT_FIELD;

begin

END_OF_SLICE := FALSE;

ACCUMULATOR.END_OF_FILE := FALSE;

ACCUMULATOR.PERIOD_SET  := FALSE;

ACCUMULATOR.INDEX_SET   := FALSE;

INDEX := TEST_PIN_RANGE'LEFT;

READ_NEXT_FIELD ( FIRST => TRUE );

while not END_OF_SLICE and not ACCUMULATOR.END_OF_FILE loop

READ_NEXT_FIELD;

end loop;

end READ_WAVE_SLICE;

--

-- END OF WAVES_OBJECTS PACKAGE BODY

--

end WAVES_OBJECTS;

**************************************************************************************************

-- full adder waves generator package spec

library WAVES_LIBRARY;

use WAVES_LIBRARY.WAVES_STANDARD.all;

use WAVES_LIBRARY.WAVES_PORT.all;

use WAVES_LIBRARY.WAVES_INTERFACE.all;

package WAVES_GENERATOR is

--

-- Specification portion of a WAVES Level 1 dataset.

--

type TEST_PINS is (X, Y, CIN, COUT, SUM );

constant PIN_CODES : STRING := "01LHXZ";

constant VALUE_DICTIONARY:EVENT_VALUE_LIST := (

EVALUE( LOW,     DRIVE, OUTPUT, CARE),

EVALUE( HIGH,    DRIVE, OUTPUT, CARE),

EVALUE( LOW,     DRIVE, INPUT,  CARE),

EVALUE( HIGH,    DRIVE, INPUT,  CARE),

EVALUE( UNKNOWN, DRIVE, OUTPUT, MASK),

EVALUE( UNKNOWN, DISCONNECTED, OUTPUT,  CARE));

constant LOGIC0 : INTEGER := 1;

constant LOGIC1 : INTEGER := 2;

constant LOGICL : INTEGER := 3;

constant LOGICH : INTEGER := 4;

constant LOGICX : INTEGER := 5;

constant LOGICZ : INTEGER := 6;

PROCEDURE WAVEFORM(signal CONNECT:inout WAVES_PORT_LIST);

end WAVES_GENERATOR;

-- full adder waves generator package body

use WORK.WAVES_OBJECTS.all;

Package Body WAVES_GENERATOR is

PROCEDURE WAVEFORM(signal CONNECT:inout WAVES_PORT_LIST) IS

constant T0 : EVENT_TIME := ETIME(0ns);

constant E0 : EVENT := (LOGIC0, T0 );

constant E1 : EVENT := (LOGIC1, T0 );

constant EL : EVENT := (LOGICL, T0 );

constant EH : EVENT := (LOGICH, T0 );

constant EX : EVENT := (LOGICX, T0 );

constant EZ : EVENT := (LOGICZ, T0 );

constant F0 : FRAME := FRAME_EVENT(E0);

constant F1 : FRAME := FRAME_EVENT(E1);

constant FL : FRAME := FRAME_EVENT(EL);

constant FH : FRAME := FRAME_EVENT(EH);

constant FX : FRAME := FRAME_EVENT(EX);

constant FZ : FRAME := FRAME_EVENT(EZ);

-- FRAME SET FOR 01LHXZ

constant FS : FRAME_SET := F0 + F1 + FL + FH + FX + FZ;

constant FSA : Frame_Set_Array := NEW_FRAME_SET_ARRAY(ALL_PINS, FS);

variable TD : TIME_DATA := NEW_TIME_DATA(FSA);

begin

APPLY_LIST (connect,(

("00000",( 20 ns, TD)),

("00101",( 20 ns, TD)),

("01001",( 20 ns, TD)),

("01110",( 20 ns, TD)),

("10001",( 20 ns, TD)),

("10110",( 20 ns, TD)),

("11111",( 20 ns, TD))

)) ;

end WAVEFORM;

end WAVES_GENERATOR;